Exp 7 - Design and Analysis of FIR Filters Using Parks–McClellan and Frequency-Sampling Techniques Using MATLAB

Program :

clc;

clear;

close all;

% EXPERIMENT 7

% Design and Analysis of FIR Filters Using

% Parks-McClellan and Frequency-Sampling Techniques

%% STEP 1: INPUT PARAMETERS

Fs = 1000;          % Sampling frequency in Hz

Fp = 100;           % Passband edge in Hz

Fst = 150;          % Stopband edge in Hz

N = 20;             % Filter order

% Normalized frequency values (1 corresponds to Fs/2)

F = [0 0.2 0.3 1];

% Desired response: passband = 1, stopband = 0

A = [1 1 0 0];

% Weighting factors for Parks-McClellan method

W = [1 10];

fprintf('===== EXPERIMENT 7 =====\n');

fprintf('Sampling frequency = %g Hz\n', Fs);

fprintf('Passband edge = %g Hz\n', Fp);

fprintf('Stopband edge = %g Hz\n', Fst);

fprintf('Filter order = %d\n', N);

fprintf('Transition width = %g Hz\n', Fst-Fp);

%% STEP 2: PARKS-MCCLELLAN DESIGN

% Equiripple FIR low-pass filter

bPM = firpm(N, F, A, W);

%% STEP 3: FREQUENCY-SAMPLING DESIGN

% fir2 designs an FIR filter from a specified

% piecewise-linear frequency response

bFS = fir2(N, F, A);

a = 1;

fprintf('\nPARKS-MCCLELLAN COEFFICIENTS:\n');

disp(bPM);

fprintf('\nFREQUENCY-SAMPLING COEFFICIENTS:\n');

disp(bFS);

%% STEP 4: FREQUENCY RESPONSES

[HPM, f] = freqz(bPM, a, 2048, Fs);

[HFS, ~] = freqz(bFS, a, 2048, Fs);

% Magnitude in dB

MPM = 20*log10(abs(HPM)+eps);

MFS = 20*log10(abs(HFS)+eps);

% Phase in degrees

PPM = unwrap(angle(HPM))*180/pi;

PFS = unwrap(angle(HFS))*180/pi;

%% STEP 5: PASSBAND RIPPLE AND STOPBAND RESPONSE

passIdx = (f <= Fp);

stopIdx = (f >= Fst);

% Passband ripple in dB

ripplePM = max(MPM(passIdx)) - min(MPM(passIdx));

rippleFS = max(MFS(passIdx)) - min(MFS(passIdx));

% Maximum stopband magnitude in dB

stopPM = max(MPM(stopIdx));

stopFS = max(MFS(stopIdx));

% Stopband attenuation in dB

attenPM = -stopPM;

attenFS = -stopFS;

fprintf('\n===== PERFORMANCE RESULTS =====\n');

fprintf('Parks-McClellan passband ripple = %.4f dB\n', ...

ripplePM);

fprintf('Frequency-sampling passband ripple = %.4f dB\n', ...

rippleFS);

fprintf('Parks-McClellan stopband attenuation = %.4f dB\n', ...

attenPM);

fprintf('Frequency-sampling stopband attenuation = %.4f dB\n', ...

attenFS);

%% STEP 6: COEFFICIENT AND PERFORMANCE COMPARISON

Method = {'Parks-McClellan'; 'Frequency-Sampling'};

PassbandRipple_dB = [ripplePM; rippleFS];

StopbandAttenuation_dB = [attenPM; attenFS];

Comparison = table(Method, PassbandRipple_dB, ...

StopbandAttenuation_dB);

disp(Comparison);

%% STEP 7: PLOT ALL GRAPHS IN ONE FIGURE

figure('Name', 'Experiment 7 - FIR Filter Comparison', ...

'NumberTitle', 'off');

% Magnitude response

subplot(2,2,1);

plot(f, MPM, 'b', 'LineWidth', 1.4);

hold on;

plot(f, MFS, 'r--', 'LineWidth', 1.4);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response');

legend('Parks-McClellan', 'Frequency-Sampling', ...

'Location', 'best');

xlim([0 Fs/2]);

ylim([-100 5]);

% Phase response

subplot(2,2,2);

plot(f, PPM, 'b', 'LineWidth', 1.4);

hold on;

plot(f, PFS, 'r--', 'LineWidth', 1.4);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Phase Response');

legend('Parks-McClellan', 'Frequency-Sampling', ...

'Location', 'best');

xlim([0 Fs/2]);

% Passband comparison

subplot(2,2,3);

plot(f, MPM, 'b', 'LineWidth', 1.4);

hold on;

plot(f, MFS, 'r--', 'LineWidth', 1.4);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Passband Comparison');

legend('Parks-McClellan', 'Frequency-Sampling', ...

'Location', 'best');

xlim([0 Fp]);

ylim([-3 1]);

% Stopband comparison

subplot(2,2,4);

plot(f, MPM, 'b', 'LineWidth', 1.4);

hold on;

plot(f, MFS, 'r--', 'LineWidth', 1.4);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Stopband Comparison');

legend('Parks-McClellan', 'Frequency-Sampling', ...

'Location', 'best');

xlim([Fst Fs/2]);

ylim([-100 5]);

fprintf('\nExperiment 7 completed.\n');

Command Window:

===== EXPERIMENT 7 =====
Sampling frequency = 1000 Hz
Passband edge = 100 Hz
Stopband edge = 150 Hz
Filter order = 20
Transition width = 50 Hz

PARKS-MCCLELLAN COEFFICIENTS:
Columns 1 through 16

0.0065 -0.0152 -0.0327 -0.0512 -0.0571 -0.0385 0.0096 0.0807 0.1572 0.2161 0.2383 0.2161 0.1572 0.0807 0.0096 -0.0385

Columns 17 through 21

-0.0571 -0.0512 -0.0327 -0.0152 0.0065

FREQUENCY-SAMPLING COEFFICIENTS:
Columns 1 through 16

0.0017 0.0017 -0.0003 -0.0074 -0.0182 -0.0213 0.0013 0.0597 0.1429 0.2178 0.2480 0.2178 0.1429 0.0597 0.0013 -0.0213

Columns 17 through 21

-0.0182 -0.0074 -0.0003 0.0017 0.0017

===== PERFORMANCE RESULTS =====
Parks-McClellan passband ripple = 3.7195 dB
Frequency-sampling passband ripple = 2.7223 dB
Parks-McClellan stopband attenuation = 33.4854 dB
Frequency-sampling stopband attenuation = 12.0480 dB

Method            PassbandRipple_dB    StopbandAttenuation_dB
    ______________________    _________________    ______________________

    {'Parks-McClellan'   }         3.7195                  33.485        
    {'Frequency-Sampling'}         2.7223                  12.048

Experiment 7 completed.

>>

Output: