Exp 8 -  Realization of FIR Filters Using Direct, Cascade and LinearPhase Structures Using MATLAB

Program:

clc;

clear;

close all;

% EXPERIMENT 8

% Realization of FIR Filters Using Direct, Cascade

% and Linear-Phase Structures Using MATLAB

%% STEP 1: INPUT PARAMETERS

Fs = 1000;       % Sampling frequency in Hz

N = 20;          % FIR filter order

L = 128;         % Number of samples

% Example FIR coefficients from a Hamming-window design.

% You can replace this line with coefficients from Exp 6 or 7.

b = fir1(N, 0.25, 'low', hamming(N+1));

a = 1;

% Unit impulse input

x = zeros(1,L);

x(1) = 1;

fprintf('===== EXPERIMENT 8 =====\n');

fprintf('Sampling frequency = %d Hz\n', Fs);

fprintf('Filter order = %d\n', N);

fprintf('Number of samples = %d\n', L);

fprintf('\nFIR coefficients:\n');

disp(b);

%% STEP 2: DIRECT REALIZATION

% Apply the FIR filter directly

yDirect = filter(b, a, x);

%% STEP 3: CASCADE REALIZATION

% Convert the FIR transfer function into second-order sections

[sos, g] = tf2sos(b, a);

% Pass the signal through each section in sequence

yCascade = x;

for k = 1:size(sos,1)

% Apply overall gain in the first section

if k == 1

sectionNumerator = g * sos(k,1:3);

else

sectionNumerator = sos(k,1:3);

end

sectionDenominator = sos(k,4:6);

yCascade = filter(sectionNumerator, ...

sectionDenominator, yCascade);

end

%% STEP 4: VERIFY COEFFICIENT SYMMETRY

% Symmetric coefficients indicate linear-phase FIR behaviour

bReverse = fliplr(b);

symmetryError = max(abs(b - bReverse));

fprintf('\n===== LINEAR-PHASE CHECK =====\n');

fprintf('Maximum coefficient symmetry error = %.12g\n', ...

symmetryError);

if symmetryError < 1e-10

fprintf('The FIR coefficients are symmetric.\n');

fprintf('The filter has linear-phase characteristics.\n');

else

fprintf('The FIR coefficients are not perfectly symmetric.\n');

end

groupDelay = N/2;

fprintf('Expected group delay = %.2f samples\n', groupDelay);

fprintf('Group delay in seconds = %.6f seconds\n', ...

groupDelay/Fs);

%% STEP 5: COMPARE REALIZATION OUTPUTS

difference = yDirect - yCascade;

maxError = max(abs(difference));

fprintf('\n===== REALIZATION COMPARISON =====\n');

fprintf('Maximum direct-cascade output error = %.12g\n', ...

maxError);

if maxError < 1e-8

fprintf('Direct and cascade realizations agree.\n');

else

fprintf('Check the cascade implementation and coefficients.\n');

end

%% STEP 6: FREQUENCY AND PHASE RESPONSE

[H,f] = freqz(b, a, 2048, Fs);

magnitudeDB = 20*log10(abs(H) + eps);

phaseDegrees = unwrap(angle(H))*180/pi;

n = 0:L-1;

%% STEP 7: DISPLAY ALL GRAPHS IN ONE FIGURE

figure('Name','Experiment 8 - FIR Realization', ...

'NumberTitle','off');

% Plot 1: Impulse responses

subplot(2,2,1);

stem(n, yDirect, 'b', 'filled');

hold on;

stem(n, yCascade, 'r--');

grid on;

xlabel('Sample index n');

ylabel('Amplitude');

title('Impulse Response');

legend('Direct','Cascade','Location','best');

% Plot 2: Output comparison

subplot(2,2,2);

plot(n, yDirect, 'b', 'LineWidth',1.4);

hold on;

plot(n, yCascade, 'r--', 'LineWidth',1.4);

grid on;

xlabel('Sample index n');

ylabel('Output');

title('Direct vs Cascade Output');

legend('Direct','Cascade','Location','best');

% Plot 3: Magnitude response

subplot(2,2,3);

plot(f, magnitudeDB, 'LineWidth',1.4);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR Magnitude Response');

xlim([0 Fs/2]);

ylim([-100 5]);

% Plot 4: Phase response

subplot(2,2,4);

plot(f, phaseDegrees, 'LineWidth',1.4);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Unwrapped Phase Response');

xlim([0 Fs/2]);

fprintf('\nExperiment 8 completed.\n');

Output:

Command Window:

===== EXPERIMENT 8 =====
Sampling frequency = 1000 Hz
Filter order = 20
Number of samples = 128

FIR coefficients:
Columns 1 through 16

0.0025 0.0026 0 -0.0086 -0.0210 -0.0242 0 0.0606 0.1446 0.2191 0.2490 0.2191 0.1446 0.0606 0 -0.0242

Columns 17 through 21

   -0.0210   -0.0086         0    0.0026    0.0025


===== LINEAR-PHASE CHECK =====
Maximum coefficient symmetry error = 0
The FIR coefficients are symmetric.
The filter has linear-phase characteristics.
Expected group delay = 10.00 samples
Group delay in seconds = 0.010000 seconds

===== REALIZATION COMPARISON =====

Maximum direct-cascade output error = 1.66533453694e-15
Direct and cascade realizations agree.

Experiment 8 completed.

>>